实验八   单周期的CPU设计

一、实验目的及环境

1、深入了解单周期CPU中指令控制器的结构和工作原理。

2、学习使用Verilog HDL设计实现单周期CPU中指令控制器。

3、装有vivado的计算机     1台

4、EGO1开发板     1块

二、实验目标及任务

1、设计和实现单周期CPU中指令控制器的结构并且进行功能仿真。

三、实验过程及记录

本节重点介绍实验的具体过程，包括：代码设计层次结构图及说明、源代码（包括注释）、PC机上进行的关键步骤截图及说明、调试过程等，这部分的内容应当与实际操作过程类似即可（简单明了）。

①电路原理图:

由实验原理图以及上述控制输出的表格可以得知，我们首先需要实现ALU_Control模块，该模块通过其中的控制信号ALUOP与指令机器码的低6位instruction[5:0]联合起来，生成运算器运算类型相对应的控制信号ALU_Control。

根据上面的实验原理图我们可以实现ALU_Control模块。之后我们需要设计CPUControl模块实现Cpu的控制。

由上述两个电路原理图可以很容易知道CpuControl模块的编写。最后通过顶层Top文件完成CPU的设计。

②源代码:

ALU_Control:

`timescale 1ns / 1ps

// ALU 控制模块

module ALU_Ctrl (

input      [1:0] ALUOp,

input      [5:0] Func,

output reg [2:0] ALU_Control

);

and and1 (temp1, ALUOp[1], Func[1]);

and and2 (temp2, Func[0], ~Func[1]);

or or1 (temp3, Func[3], temp2);

or or2 (temp4, ALUOp[0], temp1);

nand nand1 (temp5, ALUOp[1], Func[2]);

and and3 (temp6, ALUOp[1], temp3);

// ALU 控制信号输出

always @(*) begin

ALU_Control <= {temp4, temp5, temp6};

end

endmodule

SCPU_Control:

`timescale 1ns / 1ps

// CPU 控制模块

module SCPU_Ctrl (

input      [5:0] OPcode,       // OPcode

input      [5:0] Func,         // Function

output reg       RegDst,

output reg       ALUSrc_B,

output reg       MemtoReg,

output reg       Jump,

output reg       Branch,

output reg       RegWrite,

output reg [2:0] ALU_Control,

output reg       MemRead,

output reg       MemWrite

);

wire [2:0] ALU_Control_t;

// 控制信号生成

nor nor6 (RFormat, OPcode[5], OPcode[4], OPcode[3], OPcode[2], OPcode[1], OPcode[0]);

and and5b1 (LFormat_LW, OPcode[5], ~OPcode[3], ~OPcode[2], OPcode[1], OPcode[0]);

and and5b2 (LFormat_SW, OPcode[5], OPcode[3], ~OPcode[2], OPcode[1], OPcode[0]);

and and5b3 (LFormat_Beq, ~OPcode[5], ~OPcode[3], OPcode[2], ~OPcode[1], ~OPcode[0]);

and and5b4 (JFormat, ~OPcode[5], ~OPcode[3], ~OPcode[2], OPcode[1], ~OPcode[0]);

// ALU 控制

ALU_Ctrl u_ALU_Ctrl (

.ALUOp      ({RFormat, LFormat_Beq}),

.Func       (Func),

.ALU_Control(ALU_Control_t)

);

// 控制信号输出

always @(*) begin

RegDst <= RFormat;

ALUSrc_B <= LFormat_LW | LFormat_SW;

MemtoReg <= LFormat_LW;

Jump <= JFormat;

Branch <= LFormat_Beq;

RegWrite <= RFormat | LFormat_LW;

MemRead <= LFormat_LW;

MemWrite <= LFormat_SW;

ALU_Control <= ALU_Control_t;

end

// 初始化

initial begin

RegDst <= 0;

ALUSrc_B <= 0;

MemtoReg <= 0;

Jump <= 0;

Branch <= 0;

RegWrite <= 0;

ALU_Control <= 0;

MemRead <= 0;

MemWrite <= 0;

end

endmodule

Top顶层模块:

`timescale 1ns / 1ps

module top (

input  [15:0] Instrument_Select,

output        RegDst,

output        ALUSrc_B,

output        MemtoReg,

output        Jump,

output        Branch,

output        RegWrite,

output [ 2:0] ALU_Control,

output        MemRead,

output        MemWrite

);

// mem用于读取指令

reg [11:0] memory [0:1];

// SCPU_Ctrl 模块输入

reg [ 5:0] OPcode;

reg [ 5:0] Func;

// SCPU_Ctrl 模块实例化

SCPU_Ctrl u_SCPU_Ctrl (

.OPcode     (OPcode),

.Func       (Func),

.RegDst     (RegDst),

.ALUSrc_B   (ALUSrc_B),

.MemtoReg   (MemtoReg),

.Jump       (Jump),

.Branch     (Branch),

.RegWrite   (RegWrite),

.ALU_Control(ALU_Control),

.MemRead    (MemRead),

.MemWrite   (MemWrite)

);

// 读取指令到 memory

initial begin

$readmemb("Is.txt", memory);

end

// 指定执行第几条指令

reg [4:0] n = 0;

always @(Instrument_Select) begin

case (Instrument_Select)

1 << 0: n <= 0;

1 << 1: n <= 1;

endcase

OPcode <= memory[n][11:6];

Func   <= memory[n][5:0];

end

endmodule

四、实验结果分析

1.这里应给出相应的实验结果。分析应有条理，要求采用规范的书面语。

2.每个实验都需要做模拟仿真，需要对仿真波形进行简单的文字说明。

3.对下载到开发板上的图片结果做分析说明。

4.原则上要求使用图片与文字结合的形式说明，因为word和PDF文档不支持视频，所以请不要使用视频文件。

5.图片请在垂直方向，不要横向。不要用很大的图片（不要超过1M），请先做裁剪操作。

①激励文件:

`timescale 1ns / 1ps

module tb_top;

parameter XH = 2022217587;

// top 模块输入

reg  [15:0] Instrument_Select;

// top 模块输出

wire        RegDst;

wire        ALUSrc_B;

wire        MemtoReg;

wire        Jump;

wire        Branch;

wire        RegWrite;

wire [ 2:0] ALU_Control;

wire        MemRead;

wire        MemWrite;

// top 模块实例化

top u_top (

.Instrument_Select(Instrument_Select),

.RegDst           (RegDst),

.ALUSrc_B         (ALUSrc_B),

.MemtoReg         (MemtoReg),

.Jump             (Jump),

.Branch           (Branch),

.RegWrite         (RegWrite),

.ALU_Control      (ALU_Control),

.MemRead          (MemRead),

.MemWrite         (MemWrite)

);

// 依次送入第 1 ~ 16 条指令

initial begin

Instrument_Select <= 16'b0000000000000001;

#XH;

Instrument_Select <= 16'b0000000000000010;

#XH;

Instrument_Select <= 16'b0000000000000100;

#XH;

Instrument_Select <= 16'b0000000000001000;

#XH;

Instrument_Select <= 16'b0000000000010000;

#XH;

Instrument_Select <= 16'b0000000000100000;

#XH;

Instrument_Select <= 16'b0000000001000000;

#XH;

Instrument_Select <= 16'b0000000010000000;

#XH;

Instrument_Select <= 16'b0000000100000000;

#XH;

Instrument_Select <= 16'b0000001000000000;

#XH;

Instrument_Select <= 16'b0000010000000000;

#XH;

Instrument_Select <= 16'b0000100000000000;

#XH;

Instrument_Select <= 16'b0001000000000000;

#XH;

Instrument_Select <= 16'b0010000000000000;

#XH;

Instrument_Select <= 16'b0100000000000000;

#XH;

Instrument_Select <= 16'b1000000000000000;

#XH;

end

endmodule

②仿真波形:

③结果解释:

在这个激励文件的仿真波形中，使用了学号作为XH变量，依次送入指令内容，ALU_Control的输出结果与下面的表格输出内容一致。

④Instruction文件的编写:

000000100000

000000100010

000000100100

000000100101

000000100110

000000000000

000000000010

000000000011

000000001000

该文件中的部分使用上述图片中的指令形式，通过Top文件进行依次读取并在开发板中进行显示。

⑤约束文件:

# /////////////////////////////系统时钟和复位////////////////////////////////////

# set_property -dict {PACKAGE_PIN P17 IOSTANDARD LVCMOS33} [get_ports sys_clk_in ]

# set_property -dict {PACKAGE_PIN P15 IOSTANDARD LVCMOS33} [get_ports sys_rst_n  ]

# /////////////////////////////////////5个按键//////////////////////////////////////

# set_property -dict {PACKAGE_PIN R11 IOSTANDARD LVCMOS33} [get_ports {btn_pin[0]}]

# set_property -dict {PACKAGE_PIN R17 IOSTANDARD LVCMOS33} [get_ports {btn_pin[1]}]

# set_property -dict {PACKAGE_PIN R15 IOSTANDARD LVCMOS33} [get_ports {btn_pin[2]}]

# set_property -dict {PACKAGE_PIN V1  IOSTANDARD LVCMOS33} [get_ports {btn_pin[3]}]

# set_property -dict {PACKAGE_PIN U4  IOSTANDARD LVCMOS33} [get_ports {btn_pin[4]}]

# ///////////////////////////////////拨码开关sw0~sw7////////////////////////////////

set_property -dict {PACKAGE_PIN P5 IOSTANDARD LVCMOS33} [get_ports {Instrument_Select[15]}]

set_property -dict {PACKAGE_PIN P4 IOSTANDARD LVCMOS33} [get_ports {Instrument_Select[14]}]

set_property -dict {PACKAGE_PIN P3 IOSTANDARD LVCMOS33} [get_ports {Instrument_Select[13]}]

set_property -dict {PACKAGE_PIN P2 IOSTANDARD LVCMOS33} [get_ports {Instrument_Select[12]}]

set_property -dict {PACKAGE_PIN R2 IOSTANDARD LVCMOS33} [get_ports {Instrument_Select[11]}]

set_property -dict {PACKAGE_PIN M4 IOSTANDARD LVCMOS33} [get_ports {Instrument_Select[10]}]

set_property -dict {PACKAGE_PIN N4 IOSTANDARD LVCMOS33} [get_ports {Instrument_Select[9]}]

set_property -dict {PACKAGE_PIN R1 IOSTANDARD LVCMOS33} [get_ports {Instrument_Select[8]}]

# //////////////////////////////////拨码开关sw8~sw15/////////////////////////////

set_property -dict {PACKAGE_PIN U3 IOSTANDARD LVCMOS33} [get_ports {Instrument_Select[7]}]

set_property -dict {PACKAGE_PIN U2 IOSTANDARD LVCMOS33} [get_ports {Instrument_Select[6]}]

set_property -dict {PACKAGE_PIN V2 IOSTANDARD LVCMOS33} [get_ports {Instrument_Select[5]}]

set_property -dict {PACKAGE_PIN V5 IOSTANDARD LVCMOS33} [get_ports {Instrument_Select[4]}]

set_property -dict {PACKAGE_PIN V4 IOSTANDARD LVCMOS33} [get_ports {Instrument_Select[3]}]

set_property -dict {PACKAGE_PIN R3 IOSTANDARD LVCMOS33} [get_ports {Instrument_Select[2]}]

set_property -dict {PACKAGE_PIN T3 IOSTANDARD LVCMOS33} [get_ports {Instrument_Select[1]}]

set_property -dict {PACKAGE_PIN T5 IOSTANDARD LVCMOS33} [get_ports {Instrument_Select[0]}]

# //////////////////////////////////////LED0~LED15////////////////////////////////

set_property -dict {PACKAGE_PIN F6 IOSTANDARD LVCMOS33} [get_ports {RegDst}]

set_property -dict {PACKAGE_PIN G4 IOSTANDARD LVCMOS33} [get_ports {ALUSrc_B}]

set_property -dict {PACKAGE_PIN G3 IOSTANDARD LVCMOS33} [get_ports {MemtoReg}]

set_property -dict {PACKAGE_PIN J4 IOSTANDARD LVCMOS33} [get_ports {Jump}]

set_property -dict {PACKAGE_PIN H4 IOSTANDARD LVCMOS33} [get_ports {Branch}]

set_property -dict {PACKAGE_PIN J3 IOSTANDARD LVCMOS33} [get_ports {RegWrite}]

set_property -dict {PACKAGE_PIN J2 IOSTANDARD LVCMOS33} [get_ports {MemRead}]

set_property -dict {PACKAGE_PIN K2 IOSTANDARD LVCMOS33} [get_ports {MemWrite}]

set_property -dict {PACKAGE_PIN K1 IOSTANDARD LVCMOS33} [get_ports {ALU_Control[2]}]

set_property -dict {PACKAGE_PIN H6 IOSTANDARD LVCMOS33} [get_ports {ALU_Control[1]}]

set_property -dict {PACKAGE_PIN H5 IOSTANDARD LVCMOS33} [get_ports {ALU_Control[0]}]

# set_property -dict {PACKAGE_PIN J5 IOSTANDARD LVCMOS33} [get_ports {led_pin[11]}]

# set_property -dict {PACKAGE_PIN K6 IOSTANDARD LVCMOS33} [get_ports {led_pin[12]}]

# set_property -dict {PACKAGE_PIN L1 IOSTANDARD LVCMOS33} [get_ports {led_pin[13]}]

# set_property -dict {PACKAGE_PIN M1 IOSTANDARD LVCMOS33} [get_ports {led_pin[14]}]

# set_property -dict {PACKAGE_PIN K3 IOSTANDARD LVCMOS33} [get_ports {led_pin[15]}]

# ///////////////////////////////8个数码管位选信号/////////////////////////////////

# set_property -dict {PACKAGE_PIN G2 IOSTANDARD LVCMOS33} [get_ports {seg_cs_pin[0]}]

# set_property -dict {PACKAGE_PIN C2 IOSTANDARD LVCMOS33} [get_ports {seg_cs_pin[1]}]

# set_property -dict {PACKAGE_PIN C1 IOSTANDARD LVCMOS33} [get_ports {seg_cs_pin[2]}]

# set_property -dict {PACKAGE_PIN H1 IOSTANDARD LVCMOS33} [get_ports {seg_cs_pin[3]}]

# set_property -dict {PACKAGE_PIN G1 IOSTANDARD LVCMOS33} [get_ports {seg_cs_pin[4]}]

# set_property -dict {PACKAGE_PIN F1 IOSTANDARD LVCMOS33} [get_ports {seg_cs_pin[5]}]

# set_property -dict {PACKAGE_PIN E1 IOSTANDARD LVCMOS33} [get_ports {seg_cs_pin[6]}]

# set_property -dict {PACKAGE_PIN G6 IOSTANDARD LVCMOS33} [get_ports {seg_cs_pin[7]}]

# ///////////////////////////////////数码管段选信号//////////////////////////////////

# set_property -dict {PACKAGE_PIN B4 IOSTANDARD LVCMOS33} [get_ports {seg_data_0_pin[0]}]

# set_property -dict {PACKAGE_PIN A4 IOSTANDARD LVCMOS33} [get_ports {seg_data_0_pin[1]}]

# set_property -dict {PACKAGE_PIN A3 IOSTANDARD LVCMOS33} [get_ports {seg_data_0_pin[2]}]

# set_property -dict {PACKAGE_PIN B1 IOSTANDARD LVCMOS33} [get_ports {seg_data_0_pin[3]}]

# set_property -dict {PACKAGE_PIN A1 IOSTANDARD LVCMOS33} [get_ports {seg_data_0_pin[4]}]

# set_property -dict {PACKAGE_PIN B3 IOSTANDARD LVCMOS33} [get_ports {seg_data_0_pin[5]}]

# set_property -dict {PACKAGE_PIN B2 IOSTANDARD LVCMOS33} [get_ports {seg_data_0_pin[6]}]

# set_property -dict {PACKAGE_PIN D5 IOSTANDARD LVCMOS33} [get_ports {seg_data_0_pin[7]}]

# set_property -dict {PACKAGE_PIN D4 IOSTANDARD LVCMOS33} [get_ports {seg_data_1_pin[0]}]

# set_property -dict {PACKAGE_PIN E3 IOSTANDARD LVCMOS33} [get_ports {seg_data_1_pin[1]}]

# set_property -dict {PACKAGE_PIN D3 IOSTANDARD LVCMOS33} [get_ports {seg_data_1_pin[2]}]

# set_property -dict {PACKAGE_PIN F4 IOSTANDARD LVCMOS33} [get_ports {seg_data_1_pin[3]}]

# set_property -dict {PACKAGE_PIN F3 IOSTANDARD LVCMOS33} [get_ports {seg_data_1_pin[4]}]

# set_property -dict {PACKAGE_PIN E2 IOSTANDARD LVCMOS33} [get_ports {seg_data_1_pin[5]}]

# set_property -dict {PACKAGE_PIN D2 IOSTANDARD LVCMOS33} [get_ports {seg_data_1_pin[6]}]

# set_property -dict {PACKAGE_PIN H2 IOSTANDARD LVCMOS33} [get_ports {seg_data_1_pin[7]}]

⑥开发板运行结果:

根据图片不难发现实验开发板与实验所编写代码运行结果一致。

五、遇到的问题与实验心得体会

简要地叙述一下实验过程中的感受，以及其他的问题描述和自己的感想。特别是实验中遇到的困难，最后如何解决的。在用verilog代码写程序时遇到语法或其他错误，如何修改解决的。

在计算机组成实验中设计单周期CPU顶层模块是一项重要的任务。在本次实验中，我完成了这两个工作。

在本次实验中，我首先实现了ALU——Control模块，这个模块用于输出运算类型相对应的控制信号，之后进行CPU模块主体部分的编写。 在实验中，花费更多时间的是对整个CPU结构的理解，在CPU中，需要有指令寄存器、寄存器堆、译码和控制单元、ALU，还有指令的执行过程：取指、析指、执行、访存。对于CPU和指令的执行都有了更加深入的理解。在理解了CPU的结构过后，在最后的实现上，也没有花费太多的时间，只需要将其中的模块一一实现，然后使用数据线或控制线连接各个模块，就可以得到一个相对简单的CPU。